Complete Installation Guide.

# What You Are Installing

| Tool | Purpose |
| --- | --- |
| Python 3.10 | Running the pipeline |
| ArduPilot SITL | Simulated drone autopilot |
| QGroundControl | Visual ground control station |
| pymavlink | Python to drone communication |
| Groq SDK | Free LLM API |
| Docker | Portability / running on any machine |
| Git | Version control |

# PART 1 — System Preparation

## Step 1: Update your system

sudo apt update && sudo apt upgrade -y

Wait for it to finish. This can take a few minutes.

## Step 2: Install essential system tools

sudo apt install -y \

git curl wget python3 python3-pip python3-dev \

python3-venv build-essential cmake ccache g++ \

gawk make ninja-build libncurses-dev libtool \

libxml2-dev libxslt-dev python3-future python3-lxml \

python3-pexpect unzip zip

## Step 3: Verify Python is installed

python3 --version

# PART 2 — ArduPilot SITL Installation

## Step 4: Clone ArduPilot repository

cd ~

git clone https://github.com/ArduPilot/ardupilot.git

cd ardupilot

This downloads the full ArduPilot codebase. Takes 5-10 minutes.

## Step 5: Install ArduPilot dependencies

Tools/environment_install/install-prereqs-ubuntu.sh -y

When it finishes, reload your environment:

. ~/.profile

## Step 6: Build ArduCopter SITL

./waf configure --board sitl

./waf copter

## Step 7: Install MAVProxy

pip3 install MAVProxy

Verify it installed:

mavproxy.py --version

## Step 8: Test ArduPilot SITL works

cd ~/ardupilot

sim_vehicle.py -v ArduCopter --console –map

if above doesn’t work use below command

# Changed from 14550 to 14551

# so our script and QGroundControl don't compete for the same port.

# QGroundControl uses 14550 (default)

sim_vehicle.py -v ArduCopter --console --map --out udp:127.0.0.1:14551

Wait 30 seconds. You should see:

A console window with telemetry scrolling

A map window showing the drone location

The text 'EKF2 IMU0 is using GPS' in the console

# PART 3 — QGroundControl Installation

## Step 9: Download QGroundControl

cd ~

wget https://d176tv9ibo4jno.cloudfront.net/latest/QGroundControl.AppImage

## Step 10: Make it executable

chmod +x ~/QGroundControl.AppImage

## Step 11: Install required libraries for QGC

sudo apt install -y \

libgstreamer1.0-dev libgstreamer-plugins-base1.0-dev \

gstreamer1.0-plugins-bad gstreamer1.0-libav \

gstreamer1.0-gl libfuse2

## Step 12: Test QGroundControl launches

./QGroundControl-x86_64.AppImage

-  A window should open showing a map

Close it and move to next step.

## Step 13: Test QGC + SITL together

Open two terminals:

### Terminal 1 — Start SITL:

cd ~/ardupilot

sim_vehicle.py -v ArduCopter --console --map --out udp:127.0.0.1:14551

Wait for: EKF2 IMU0 is using GPS

### Terminal 2 — Start QGC:

./QGroundControl-x86_64.AppImage

- QGC should auto-connect and show the drone on the map

-  Top bar should show 'Ready To Fly' (not 'Comms Lost')

# PART 4 — Python Dependencies

## Step 14: Install project Python packages

pip3 install groq==0.9.0 pymavlink==2.4.41 jsonschema==4.23.0 httpx==0.27.2

## Step 15: Verify each package installed correctly

python3 -c "import groq; print('groq ok')"

python3 -c "from pymavlink import mavutil; print('pymavlink ok')"

python3 -c "import jsonschema; print('jsonschema ok')"

python3 -c "import httpx; print('httpx ok')"

-  All four should print ok

# PART 5 — Groq API Key (Free LLM)

## Step 16 free Groq API key

1. Go to https://console.groq.com

2. Sign up with Google or email — completely free

3. Click API Keys in the left sidebar

4. Click Create API Key

5. Copy the key (starts with gsk_...)

## Step 17: Set key permanently

echo 'export GROQ_API_KEY="paste-your-key-here"' >> ~/.bashrc

source ~/.bashrc

## Step 18: Verify key is set

echo $GROQ_API_KEY

-  Should print your key starting with gsk_

## Step 19: Test Groq API works

python3 -c "

import os

from groq import Groq

client = Groq(api_key=os.environ.get('GROQ_API_KEY'))

r = client.chat.completions.create(

model='llama-3.3-70b-versatile',

messages=[{'role':'user','content':'say hello'}],

max_tokens=10

)

print('Groq working:', r.choices[0].message.content)

"

-  Should print: Groq working: Hello!

# PART 6 — Docker Installation

## Step 20: Install Docker

sudo apt install docker.io -y

## Step 21: Add yourself to docker group

sudo usermod -aG docker $USER

newgrp docker

## Step 22: Fix socket permissions

sudo chmod 666 /var/run/docker.sock

## Step 23: Verify Docker works

docker --version

-  Should print Docker version 29.x.x

docker run hello-world

-  Should print Hello from Docker!

# PART 7 — Project Setup

## Step 24: Clone the project

cd ~

git clone https://github.com/ojassannak/Drone_Task_1.git

cd omokai-drone-task

## Step 25: Verify all project files exist

ls -la

You should see all of these files:

Dockerfile  requirements.txt  schema.json

llm_planner.py  validator.py  executor.py

main.py  README.md  CITATIONS.md  INSTALL.md

## Step 26: Build the Docker image

docker build -t omokai-drone .

-  Should end with: Successfully tagged omokai-drone:latest

# PART 8 — Running the Pipeline

## Step 27: Start SITL (Terminal 1)

cd ~/ardupilot

sim_vehicle.py -v ArduCopter --console --map --out udp:127.0.0.1:14551

Wait for: EKF2 IMU0 is using GPS

## Step 28: Start QGroundControl (Terminal 2)

~/QGroundControl.AppImage

Wait for drone to appear on map.

## Step 29: Run a mission (Terminal 3)

cd ~/omokai-drone-task-1

docker run -it \

-e GROQ_API_KEY=$GROQ_API_KEY \

--network host \

omokai-drone \

python3 main.py "Patrol a 30 metre square loop twice at 10 metres altitude"

# PART 9 — Verify Everything Works

## Test 1: Valid mission (drone should fly)

python3 main.py "Patrol a 30 metre square loop twice at 10 metres altitude"

-  Drone arms, takes off, flies square, lands

## Test 2: Validator rejects bad altitude

python3 main.py "Patrol a square at 80 metres altitude"

-  Should print: MISSION REJECTED: altitude 80m exceeds ceiling 50m

-  Drone should NOT move

## Test 3: LLM rejects nonsense

python3 main.py "Make me a coffee"

-  Should print: MISSION REJECTED

-  Drone should NOT move

# Troubleshooting the bug which i face during this task and how to fix it

| Problem | Fix |
| --- | --- |
| GROQ_API_KEY is empty | Run: source ~/.bashrc then echo $GROQ_API_KEY |
| Docker permission denied | Run: sudo chmod 666 /var/run/docker.sock |
| SITL not connecting | Make sure SITL started with --out udp:127.0.0.1:14551 |
| QGC shows Comms Lost | Script is on port 14550, change to 14551 in executor.py |
| Drone not moving in QGC | Restart SITL with --out flag, reconnect QGC first |
| model decommissioned error | Change model to llama-3.3-70b-versatile in llm_planner.py |
| Docker image not found | Run: docker build -t omokai-drone . from project folder |